6502 ASM Codes

Written by Barret

Well I don't know much about ASM myself, but I did say if I ever find anything out or start to understand it , I'll post about it. Well Here it is !! Have fun Hacking!

*Note that I optained this information from a book on

personal computers,that use the 6502

processor

Well theis first part is taken from the book that I am using written by Ken Skier.

So here it is..

This is the Mnemonic List:

ADC = add memory to accumulator with carry

AND = "AND" memory with accumulator

ASL = shift left one bit (memory or accumulator)

BCC = branch on carry clear

BCS  = branch on carry set

BEQ = branch on result zero

BIT   = test bits in memorywith accumulator

BMI  = Branch on result minus

BML = branch on result not zero

BPL = branch onresult plus

BVC = branch on overflow clear

BVS = branch on overflow set

CLC = clear carry flag

CLD = clear decimal mode

CLI = clear interrupt disable bit

CLV = clear overflow flag

CMP = compare memory and accumulator

CPX = compare memory and "X" register

CPY = compare memory and "Y" register

DEC = decrement memory

DEX = decrement register "X"

DEY = decrement register "Y"

EOR = "EXCLUSIVE OR" memory with accumulator

INC = increment memorty

INX = increment register X

INY = increment register Y

JMP = jump to new location

JSR = jump to new location saving return address

LDA = load accumulator with memory

LDX = load register X with memory

LDY = load register Y with memory

LSR = shift right one bit ( memory or accumulor)

NOP = no operation

ORA = "OR" memory with accumulor

PHA = push accumulator on stack

PHP = push processor status on stack

PLA = pull accumulator from from stack

PLP = pull processor status from stack

ROL = rotate one bit left ( memory or accumulator)

ROR = rotate one bit right ( memory or accumulator)

RTI = return from interrupt

RTS = return from subroutine

SBC = subtract memory from accumulator with borrow

SEC = set carry flag

SED = set decimal mode

SEI = set interrupt disable status

STA = store accumulator in memory

STX = store register "X" in memory

STY = store register "Y" in memory

TAX = transfer accumulator to register "X"

TAY = transfer accumulator to register "Y"

TSX = transfer stack pointer to register "X"

TXA = transfer "X" to accumulator

TXS = transfer register "X" to stack pointer

TYA = transfer register "Y" to accumulator

This is the Opcode List:

00=BRK

01=ORA

02=Future Expansion

03=Future Expansion

04=Future Expansion

05=ORA -- ZERO PAGE

06=ASL -- ZERO PAGE

07=Future Expansion

08=PHP

09=ORA -- IMMEDIATE

0A=ASL -- ACCUMULATOR

0B=Future Expansion

0C=Future Expansion

0D=ORA -- ACSOLUTE

0F=Future Expansion

10=BPL

11=ORA -- (INDIRECT),Y

13=Future Expansion

14=Future Expansion

15=ORA -- ZERO PAGE

16=ASL -- ZERO PAGE, X

17=Future Expansion

18=CLC

19=ORA -- ABSOLUTE , Y

1A=Future Expansion

1B=Future Expansion

1C=Future Expansion

1D=ORA -- ABSOLUTE, X

1E=Future Expansion

1F=Future Expansion

20=JSR

21=AND -- (INDIRECT,X)

22=Future Expansion

23=Future Expansion

24=BIT -- ZERO PAGE

25=AND -- ZERO PAGE

26=ROL -- ZERO PAGE

27=Future Expansion

28=PLP

29=AND -- IMMEDIATE

2A=ROL -- ACCUMULATOR

2B=Future Expansion

2C=BIT -- ABSOLUTE

2D=AND -- ABSOLUTE

2E=ROL -- ABSOLUTE

2F=Future Expansion

30=BMI

31=AND -- (INDIRECT,X)

32=Future Expansion

33=Future Expansion

34=Future Expansion

35=AND -- ZERO PAGE,X

36=ROL -- ZERO PAGE,X

37=Future Expansion

38=SEC

39=AND -- ABSOLUTE,Y

3A=Future Expansion

3B=Future Expansion

3C=Future Expansion

3D=AND -- ABSOLUTE,X

3F=Future Expansion

40=RTI

41=EOR -- (INDIRECT,X)

42=Future Expansion

43=Future Expansion

44=Future Expansion

45=EOR -- ZERO PAGE

46=LSR -- ZERO PAGE

47=Future Expansion

48=PHA

49=EOR --IMMEDIATE

4A=LSR -- ACCUMULATOR

4B=Future Expansion

4C=JMP --ABSOLUTE

4D=EOR --ABSOLUTE

4E=LSR --ABSOLUTE

4F=Future Expansion

50=BVC

51=EOR --(INDIRECT),Y

52=Future Expansion

53=Future Expansion

54=Future Expansion

55=EOR --ZERO PAGE,X

56=ZERO PAGE,X

57=Future Expansion

58=CLI

59=EOR --ABSOLUTE,Y

5A=Future Expansion

5B=Future Expansion

5C=Future Expansion

5D=EOR -- ABSOLUTE,X

5E=LSR --ABSOLUTE,X

5F=Future Expansion

60=RTS

61=ADC -- (INDIRECT,X)

62=Future Expansion

63=Future Expansion

64=Future Expansion

65=ADC -- ZERO PAGE

66=ROR -- ZERO PAGE

67=Future Expansion

68=PLA

69=ADC -- IMMEDIATE

6A=ROR -- ACCUMULATOR

6B=Future Expansion

6C=JMP -- INDIRECT

6D=ADC -- ABSOLUTE

6E=ROR -- ABSOLUTE

6F=Future Expansion

70=BVS

71=ADC -- (INDIRECT),Y

72=Future Expansion

73=Future Expansion

74=Future Expansion

75=ADC -- ZERO PAGE,X

76=ROR-- ZERO PAGE,X

77=Future Expansion

78=SEI

79=ADC-- ABSOLUTE,Y

7A=Future Expansion

7B=Future Expansion

7C=Future Expansion

7D=ADC -- ABSOLUTE,X

7E=ROR -- ABSOLUTE,X

7F=Future Expansion

80=Future Expansion

81=STA -- (INDIRECT,X)

82=Future Expansion

83=Future Expansion

84=STY -- ZERO PAGE

85=STA -- ZERO PAGE

86=STX -- ZERO PAGE

87=Future Expansion

88=DEY

89=Future Expansion

8A=TXA

8B=Future Expansion

8C=STY -- ABSOLUTE

8D=STA -- ABSOLUTE

8E=STX -- ABSOLUTE

8F=Future Expansion

90=BCC

91=STA -- (INDIRECT),Y

92=Future Expansion

93=Future Expansion

94=STY -- ZERO PAGE,X

95=STA -- ZERO PAGE,X

96=STX -- ZERO PAGE,Y

97=Future Expansion

98=TYA

99=STA -- ABSOLUTE,Y

9A=TXS

9B=Future Expansion

9C=Future Expansion

9D=STA -- ABSOLUTE,X

9E=Future Expansion

9F=Future Expansion

A0=LDY -- IMMEDIATE

A1=LDA -- (INDIRECT,X)

A2=LDX -- IMMMEDIATE

A3=Future Expansion

A4=LDY -- ZERO PAGE

A5=LDA -- ZERO PAGE

A6=LDX -- ZERO PAGE

A7=Future Expansion

A8=TAY

A9=LDA -- IMMEDIATE

AA=TAX

AB=Future Expansion

AC=LDY -- ABSOLUTE

AD=LDA -- ABSOLUTE

AE=LDX -- ABSOLUTE

AF=Future Expansion

B0=BCS

B1=LDA -- (INDIRECT),Y

B2=Future Expansion

B3=Future Expansion

B4=LDY -- ZERO PAGE,X

B5=LDA -- ZERO PAGE,X

B6=LDX -- ZERO PAGE,Y

B7=Future Expansion

B8=CLV

B9=LDA -- ABSOLUTE,Y

BA=TSX

BB=Future Expansion

BC=LDY -- ABSOLUTE,X

BD=LDA -- ABSOLUTE,X

BE=LDX -- ABSOLUTE,Y

BF=Future Expansion

C0=CPY -- IMMEDIATE

C1=CMP -- (INDIRECT),Y

C2=Future Expansion

C3=Future Expansion

C4=CPY -- ZERO PAGE

C5=CMP -- ZERO PAGE

C6=DEC -- ZERO PAGE

C7=Future Expansion

C8=INY

C9=CMP -- IMMEDIATE

CA=DEX

CB=Future Expansion

CC=CPY -- ABSOLUTE

CD=CMP -- ABSOLUTE

CE=DEC -- ABSOLUTE

CF=Future Expansion

D0=BNE

D1=CMP -- (INDIRECT),Y

D2=Future Expansion

D3=Future Expansion

D4=Future Expansion

D5=CMP -- ZERO PAGE,X

D6=DEC -- ZERO PAGE,X

D7=Future Expansion

D8=CLD

D9=CMP -- ABSOLUTE,Y

DA=Future Expansion

DB=Future Expansion

DC=Future Expansion

DD=CMP -- ABSOLUTE,X

DE=DEC -- ABSOLUTE,X

DF=Future Expansion

E0=CPX -- IMMEDIATE

E1=SEC -- (INDIRECT,X)

E2=Future Expansion

E3=Future Expansion

E4=CPX -- ZERO PAGE

E5=SBC -- ZERO PAGE

E6=ZERO PAGE

E7=Future Expansion

E8=INX

E9=SBC -- IMMEDIATE

EA=NOP

EB=Future Expansion

EC=CPX -- ABSOLUTE

ED=SBC -- ABSOLUTE

EE=INC -- ABSOLUTE

EF=Future Expansion

F0=BEQ

F1=SBC -- (INDIRECT),Y

F2=Future Expansion

F3=Future Expansion

F4=Future Expansion

F5=SBC -- ZERO PAGE,X

F6=INC -- ZERO PAGE,X

F7=Future Expansion

F8=SED

F9=SBC -- ABSOLUTE,Y

FA=Future Expansion

FB=Future Expansion

FC=Future Expansion

FD=SBC -- ABSOLUTE,X

FE=INC -- ABSOLUTE,X

FF=Future Expansion

Good luck to all the hackers that want to learn ASM.. I find it hard , but I am starting to understand it.. To all that I may have offended, sorry man !! Now I see your point!! later !

AGAIN THIS INFORMATION WAS OPTAIN FROM KEN SKIER  ..Book Called: Beyond Games..